Microprocessor and Computer Architecture

UE21CS251B

4th Semester, Academic Year 2022-23

Date:

| Name: | SRN: | Section |
| --- | --- | --- |

Week#____2_______            Program Number: ____1___

Title of the Program

Write a program in ARM7TDMI-ISA to copy a block of N data items from Location A to Location B.

a. Use Full word (.word directive)

b. Use Half word(.hword directive)

c.  Use Byte wise (.Byte directive)

- ARM Assembly Code

a.

.data

a: .word 53,72,13,38,62

b: .word 0,0,0,0,0

.text

ldr r0,=a

ldr r1,=b

mov r3,#5

loop: ldr r4,[r0]

add r0,r0,#4

str r4,[r1]

add r1,r1,#4

subs r3,r3,#1

bne loop

.end

b.

.data

a: .hword 21,53,11,7,4

b: .hword 0,0,0,0,0

.text

ldr r0,=a

ldr r1,=b

mov r3,#5

loop: ldrh r4,[r0]

add r0,r0,#2

strh r4,[r1]

add r1,r1,#2

subs r3,r3,#1

bne loop

.end

c.

.data

a: .byte 21,53,11,7,4

b: .byte 0,0,0,0,0

.text

ldr r0,=a

ldr r1,=b

mov r3,#5

loop: ldrb r4,[r0]

add r0,r0,#1

strb r4,[r1]

add r1,r1,#1

subs r3,r3,#1

bne loop

.end

- Output Screen Shots  (Three)

The output should be verified for word, half word, byte

Microprocessor and Computer Architecture

UE21CS251B

4th Semester, Academic Year 2022-23

Date:

| Name: | SRN: | Section |
| --- | --- | --- |

Week#____2_______            Program Number: ____2___

Title of the Program

Write a program in ARM7TDMI-ISA to find the sum of  N data items in the memory. Store the result in the memory location.

a. Use Full word (.word directive)

b. Use Half word(.hword directive)

c.  Use Byte wise (.Byte directive)

I.ARM Assembly Code

a.

.data

a: .word 23,53,32,54,21,43,76,32,56,11

sum: .word 0

.text

ldr r0,=a

ldr r4,=sum

mov r1,#10

mov r3,#0

loop:

ldr r2,[r0]

add r3,r3,r2

add r0,r0,#4

sub r1,r1,#1

cmp r1,#0

bne loop

str r3,[r4]

swi 0x11

b.

.data

a: .hword 22,12,10,23,76,4,35,32,59,87

sum: .word 0

.text

ldr r0,=a

ldr r4,=sum

mov r1,#10

mov r3,#0

loop:

ldrh r2,[r0]

add r3,r3,r2

add r0,r0,#2

sub r1,r1,#1

cmp r1,#0

bne loop

str r3,[r4]

swi 0x11

c.

.data

a: .byte 43,12,4,21,14,22,15,2,8,28

sum: .word 0

.text

ldr r0,=a

ldr r4,=sum

mov r1,#10

mov r3,#0

loop:

ldrb r2,[r0]

add r3,r3,r2

add r0,r0,#1

sub r1,r1,#1

cmp r1,#0

bne loop

str r3,[r4]

swi 0x11

II. Output Screen Shots (Three)

The output should be verified for word, half word, byte

Microprocessor and Computer Architecture

UE21CS251B

4th Semester, Academic Year 2022-23

Date:

| Name: | SRN: | Section |
| --- | --- | --- |

Week#____2_______            Program Number: ____3___

Title of the Program

Write a program in ARM7TDMI-ISA to find the sum of  N natural numbers.  Store the result in the memory location.

I.ARM Assembly Code

.data

a: .word 0

.text

ldr r2,=a

mov r1,#13

mov r0,#0

mov r3,#1

loop:

add r0,r0,r3

add r3,r3,#1

cmp r3,r1

bne loop

str r0,[r2]

swi 0x11

II. Output Screen Shots (One)

Microprocessor and Computer Architecture

UE21CS251B

4th Semester, Academic Year 2022-23

Date:

| Name: | SRN: | Section |
| --- | --- | --- |

Week#____2_______            Program Number: ____4___

Title of the Program

Write a program in ARM7TDMI-ISA to find the product of two 32bit numbers using barrel shifter.

I.ARM Assembly Code

Code to multiply a number by 25

mov r0,#100

mov r1,r0,lsl#5

rsb r0,r0,r0,lsl#3

sub r0,r1,r0

II. Output Screen Shot  (One)

Microprocessor and Computer Architecture

UE21CS251B

4th Semester, Academic Year 2022-23

Date:

| Name: | SRN: | Section |
| --- | --- | --- |

Week#____2_______            Program Number: ____5___

Title of the Program

Convert the following statement in C language into an ALP using ARM7TDMI – ISA.

IF([A]==[B]) then C=[A]+[B];

ELSE IF ([B]==[C]) D=[A]-[B];

ELSE E=[A]*[B]

Where A,B C, D & E are memory locations.

I.ARM Assembly Code

.data

a: .word 23

b: .word 23

c: .word 13

d: .word 37

e: .word 21

.text

ldr r0,=a

ldr r1,=b

ldr r2,=c

ldr r3,=d

ldr r4,=e

ldr r6,[r0]

ldr r7,[r1]

ldr r11,[r2]

cmp r6,r7

beq addi

cmp r7,r11

beq subt

mul r5,r6,r7

str r4,[r5]

swi 0x11

addi:

add r8,r6,r7

str r8,[r3]

swi 0x11

subt:

sub r9,r0,r1

str r9,[r3]

swi 0x11

II. Output Screen Shot  (One)

Microprocessor and Computer Architecture

UE21CS251B

4th Semester, Academic Year 2022-23

Date:

| Name: | SRN: | Section |
| --- | --- | --- |

Week#____2_______            Program Number: ____6__

Title of the Program

Write a program in ARM7TDMI-ISA to find the factorial of a number.

I.ARM Assembly Code

.text

mov r0,#8

mov r1,r0

loop:

sub r0,r0,#1

mul r2,r0,r1

mov r1,r2

cmp r0,#1

bne loop

swi 0x11

- Output Screen Shot  (One)

Disclaimer:

- The programs and output submitted is duly written, verified and executed by me.

- I have not copied from any of my peers nor from the external resource such as internet.

- If found plagiarized, I will abide with the disciplinary action of the University.

Signature:

Name:

SRN:

Section:

Date: