Microprocessor and Computer Architecture

UE21CS251B

4th Semester, Academic Year 2022-23

Date:

| Name: | SRN: | Section |
| --- | --- | --- |

Week#____4_______            Program Number: ____1__

Title of the Program

Write an ALP to add two 64-bit numbers loaded from memory and store the result in memory.

I.ARM Assembly Code

.data

a: .word 23127437,32287784

b: .word 12345678,87654321

sum: .word 0,0

.text

ldr r0, =a

ldr r1, =b

ldr r2, =sum

ldr r3, [r0],#4

ldr r4, [r1],#4

add r3, r3, r4

str r3, [r2],#4

ldr r3, [r0]

ldr r4, [r1]

add r3, r3, r4

str r3, [r2]

swi 0x11

.end

II. Output Screen Shots (One)

Microprocessor and Computer Architecture

UE21CS251B

4th Semester, Academic Year 2022-23

Date:

| Name: | SRN: | Section |
| --- | --- | --- |

Week#____4______            Program Number: ____2___

Title of the Program

Write an ALP to find 1’s and 2’s complement of a 32-bit number

I.ARM Assembly Code

.text

ldr r0,=0x12345678

mvn r1,r0

add r2,r1,#1

swi 0x11

.end

II. Output Screen Shots (One)

Microprocessor and Computer Architecture

UE21CS251B

4th Semester, Academic Year 2022-23

Date:

| Name: | SRN: | Section |
| --- | --- | --- |

Week#____4_______            Program Number: ____3__

Title of the Program

Write an ALP to scan a 32-bit number if it is negative or positive

I.ARM Assembly Code

.text

mov r0,#-83

cmp r0,#0

bmi neg

b pos

neg:

mov r1,#1

b end

pos:

mov r1,#0

end:

swi 0x11

.end

II. Output Screen Shot  (One)

Microprocessor and Computer Architecture

UE21CS251B

4th Semester, Academic Year 2022-23

Date:

| Name: | SRN: | Section |
| --- | --- | --- |

Week#____4_______            Program Number: ____4___

Title of the Program

Write an ALP to find the number of  zeroes, positive and negative numbers in a given array

I.ARM Assembly Code

.data

a: .word 42,7,0,-3,0,-54,23,-11,33

.text

ldr r0,=a

mov r1,#0  ;number of zeroes

mov r2,#0  ;number of positive numbers

mov r3,#0  ;number of negative numbers

mov r5,#9

loop:

ldr r4,[r0],#4

cmp r4,#0

beq zero

cmp r4,#0

bgt positive

blt negative

zero:

add r1,r1,#1

b next

positive:

add r2,r2,#1

b next

negative:

add r3,r3,#1

next:

sub r5,r5,#1

cmp r5,#0

bne loop

swi 0x11

.end

II. Output Screen Shot  (One)

Microprocessor and Computer Architecture

UE21CS251B

4th Semester, Academic Year 2022-23

Date:

| Name: | SRN: | Section |
| --- | --- | --- |

Week#____4_______            Program Number: ____5__

Title of the Program

Write an ALP to count the number of 1’s and 0’s in a given 32-bit number

I.ARM Assembly Code

.text

ldr r0,=0b11100110010101001101100101101101

mov r1,#32

mov r2,#0 ;count of 1s

mov r3,#0 ;count of 0s

loop:

and r4,r0,#1

cmp r4,#1

beq one

b zero

one:

add r2,r2,#1

mov r0,r0,lsr #1

sub r1,r1,#1

cmp r1,#0

bne loop

swi 0x11

zero:

add r3,r3,#1

mov r0,r0,lsr #1

sub r1,r1,#1

cmp r1,#0

bne loop

swi 0x11

.end

II. Output Screen Shot  (One)

Microprocessor and Computer Architecture

UE21CS251B

4th Semester, Academic Year 2022-23

Date:

| Name: | SRN: | Section |
| --- | --- | --- |

Week#____4_______            Program Number: ____6__

Title of the Program

Write an ALP to check the given number has odd or even number of 1’s and display the result. (Even Parity and Odd Parity)

I.ARM Assembly Code

.data

o: .asciiz "Odd parity"

e: .asciiz "Even parity"

.text

ldr r5,=0b11100110010101001101100101100101

mov r1,#32

mov r2,#0 ;count of 1s

mov r3,#0 ;count of 0s

loop:

and r4,r5,#1

cmp r4,#1

beq one

b zero

one:

add r2,r2,#1

mov r5,r5,lsr #1

sub r1,r1,#1

cmp r1,#0

bne loop

b print

zero:

add r3,r3,#1

mov r5,r5,lsr #1

sub r1,r1,#1

cmp r1,#0

bne loop

print:

and r6,r2,#1

cmp r6,#1

beq odd

b even

odd:

ldr r0,=o

swi 0x02

swi 0x11

even:

ldr r0,=e

swi 0x02

swi 0x11

.end

II. Output Screen Shot

(Two-One for odd Parity, one for even Parity)

Microprocessor and Computer Architecture

UE21CS251B

4th Semester, Academic Year 2022-23

Date:

| Name: | SRN: | Section |
| --- | --- | --- |

Week#____2_______            Program Number: ____7__

Title of the Program

Write an ALP to perform multiplication using shift method (without using MUL)

I.ARM Assembly Code

Code to multiply a number by 25

mov r0,#100

mov r1,r0,lsl#5

rsb r0,r0,r0,lsl#3

sub r0,r1,r0

II. Output Screen Shot  (One)

Disclaimer:

- The programs and output submitted is duly written, verified and executed by me.

- I have not copied from any of my peers nor from the external resource such as internet.

- If found plagiarized, I will abide with the disciplinary action of the University.

Signature:

Name:

SRN:

Section:

Date: